GENERAL

summary

Section Includes:

edit note: delete the following panel types not required for project.

Smooth, flat, direct apply, glue-up wall or ceiling panels.

Standard square, rectangular or custom size, shape and/or thickness.

Related Sections:

edit note: add or delete sections per project requirements.

Section 09 53 00 – Acoustical Ceiling Suspension Systems.

Section 09 51 00 – Acoustical Ceilings.

Section 09 21 16 – Gypsum Board Assemblies.

Division 21 – Fire Suppression.

Division 23 – Mechanical. Diffusers, vents, and other mechanical items.

Division 26 – Electrical. Lights and other ceilings mounted electrical items.

system description

Acoustic Performance Requirements:

NRC (Noise Reduction Coefficient): Comply with ASTM C423-90a.

SUBMITTALS

Comply with Section 01 33 00 – Submittal Procedures.

Product Data: Manufacturer’s technical data for each type of panel including fire-resistive characteristics, finishes, details of installation, and the following:

Manufacturer's installation instructions.

Certified test reports indicating compliance with Performance Requirements specified herein.

Samples: 2 full size sets of samples of each specified panel for color selection or verification.

Closeout Submittals: Comply with Section 01 77 00 – Closeout Procedures.

Operating and Maintenance Manual, including cleaning and maintenance instructions.

Extra Material for Owner’s stock.

Material Safety Data Sheets (MSDS).

QUALITY ASSURANCE

Single Source Responsibility: Obtain panel units for entire Project from a single manufacturer.

Manufacturer's Qualifications: Firm with not less than Closeout Submittals: Comply with Section 5 years’ experience in manufacturing products similar in complexity to those required for this Project.

Installer's Qualifications: Firm with not less than 5 years’ experience in direct apply, glue-up installation of acoustic products similar in complexity to those required for this Project, including specific requirements indicated.

Successfully completed not less than 5 comparable scale projects.

DELIVERY, STORAGE, AND HANDLING

Comply with Section 01 60 00 – Product Requirements.

Deliver and store materials flat in manufacturer's original unopened containers with brands, names, and production lot numbers clearly marked on these containers.

Storage and Protection: Comply with manufacturer's recommendations.

Store products flat in a cool, temperature-humidity controlled space

Protect from damp, water-soaked conditions, any possible damage.

PROJECT CONDITIONS

Environmental Requirements within the building:

Keep panels flat, let ISOTEX acoustic products acclimate to controlled room temperatures prior to installation to achieve best finished results.

Avoid extreme temperatures, high humidity, dirt and dust; it’s essential to have rooms and substrates prepped as clean for best adhesion.

scheduling

Do not install acoustic ceilings until work in plenum space is completed, tested and approved.

Do not install until areas intended for installation have been properly cleaned and prepped.

WARRANTY

Provide manufacturer's written warranty per Section 01 77 00 – Closeout Procedures.

Maintenance

Extra Materials:

Deliver not less than the following quantity of each type, color, surface, edge or pattern of material, exclusive of material required to properly complete installation.

Edit Note: retain amount of extra materials used on project.  delete this article If no extra materials required or acceptable.

3 percent.

5 percent.

1 carton.

Furnish Extra Materials from same production run to verify color run.

Package replacement materials with protective covering, identified with appropriate labels.

PRODUCTS

MANUFACTURERS

Subject to compliance with requirements, provide products from the following manufacturer:

- Texnov Isotex acoustic

839 Joseph-Louis-Mathieu

Sherbrooke, Québec

Canada J1R 0X3

Telephone: 1 877 316-6388 toll-free customer service/technical support

1 819 820-1188

Website:		www.texnov.com

E-mail:		info@texnov.com

Substitutions: Comply with Section 01 60 00 – Product Requirements.

manufactured units

edit note: select the following system for adhesive applied panels.

Acoustic ISOTEX Flat, smooth surface Direct Apply, Glue-up Wall or Ceiling Panels: Acoustical foam panel consisting of lightweight, open-cell ISOTEX foam core, with or without applied HPC finish surface.

ISOTEX foam physical properties

Density: 0.5 to 0.7 pounds per cubic foot (0.23 to 0.32 kg per m3).

Tensile Strength: 8 psi (0.06 MPa).

Flammability: Class 1 per ASTM E84

Flame Spread : Natural : 5

Smoke Density : Natural : 50

Flammability: CAN ULCS-102

Flame Spread : Natural : 0

Smoke Density : Natural : 30

Size:  standard square or rectangular sizes up to 48” x 96”

edit note: retain the following panel thickness for ISOTEX

Panel Thickness: typical 1-to-4 inch or custom; consult Isotex to develop custom sizes, shapes edge details, etc., to suit specific project requirements

Edit Note: select panel color below. delete panel colors not required for project.

NATURAL

Finish: Natural White.

Finish: Natural Grey.

Finish: Natural Black

Sound Absorption Coefficients: Type B and A mountings, ASTM C423-90a. (Natural)

- Frequencies (Hz)	125	250	500	1,000	2,000	4,000	NRC

1-inch thickness: 	0.13	0.25	0.60	0.85	0.96	0.95	0.65

1-1/2-inch thickness:	0.08	0.29	0.73	0.94	0.97	0.89	0.75

2-inch thickness:	0.19	0.55	1.17	1.21	1.15	1.13	1.00

2-1/2-inch thickness:	0.19	0.62	1.15	1.21	1.14	1.2	1.05

3-inch thickness	0.09	0.68	1.2	1.18	1.12	1.05	1.05

Acceptable Product: ISOTEX smooth, flat ‘direct apply, glue-up’ wall or ceiling panels

Direct apply adhesive SYSTEM

Adhesive: Non-toxic, water- based adhesive, for use with all ISOTEX products.

Isotex acoustic adhesive 10.5 oz. tube cartridges, 5-gallon bucket roll-on or approved substitute.

EXECUTION

EXAMINATION

Verification of Conditions: Examine areas and conditions under which work is to be performed and identify conditions detrimental to proper and or timely completion; consult Isotex acoustic

Do not proceed until unsatisfactory conditions have been corrected.

edit note: Use the following preparation article for adhesive applied installations only.

preparation

Prior to installing direct apply, wall or ceiling panels, make certain that substrate surfaces to which panels will be applied are smooth, clean and level; free of dust, dirt, and other residues or contaminants which could inhibit a strong bond to develop.

INSTALLATION

Comply with manufacturers’ instructions and recommendations for installation of acoustic panels:

Coordinate with mechanical, electrical and fire concerning pipe and fixture locations and spacing, etc., positioned about walls or ceilings.

Lay out pattern per reflected ceiling drawings.

Refer to manufacturer’s written installation instructions, phone pinta for any additional project specific concerns.

edit note: use the following three paragraphs for adhesive applied installation.

Cut adhesive tube tips back to produce 1/4-inch diameter (6.4 mm) bead flow.

Use clean, thin white cotton gloves to handle panels.

Apply adhesive to panel backs per manufacturer's recommended X pattern and installation instructions; press panels firmly into place and smooth evenly across them to ensure a strong lasting bond; adhesive tack should be immediate.

Install panels true to lines, position and plane as indicated on project drawings.

CLEANing

Clean up using soap and water; use clean, soft water-dampened cotton rags to wet wipe and remove unwanted adhesive from any adjacent surfaces. Lightly brush or wet wipe ‘pat and dab’ panel faces or edges to remove unwanted fingerprints, dust or adhesive marks as soon as possible.

Panel faces and edges may also be vacuumed using soft bristle brush attachments or lightly dusted using Swiffer® Dusters.

Remove and reinstall improperly installed, damaged or discolored panels or ones that cannot be properly cleaned and install new material.

END OF SECTION

ISOTEX™ is a registered product of Texnov